You and Your Septic System

How It Works

- Waste water goes from your house into a septic tank

- Heavy solids sink to the bottom (bacteria cannot keep up with accumulation, which results in a layer of sludge)

- Lighter solids float to the top (bacteria breaks down the solids in this layer)

- The liquid layer goes through a filter (if installed) out of the tank to an absorption field

Operating your system

- Use water conservatively, repair leaking sinks and toilets

- DO NOT drain runoff rain water into system

- DO NOT put toxic chemicals or cat litter into the system

- DO NOT put grease or coffee grounds into the system

- DO NOT put paper products (excluding toilet paper) into the system

- CONSERVATIVE use of a garbage disposal is acceptable, but adds a lot of solids to the system, which may increase the need to pump out your tank

- Using normal household cleaners is okay if used as directed
If a water softener is used, limit the regeneration frequency

- Avoid septic tank treatments IE: Rid-X, & Roebic; according to the Iowa State Extension Service, there is no scientific proof that these treatments work.

Maintaining your system

- Install a tank outlet filter to help trap solids

- Protect the absorption field: no heavy vehicles; use grass or plants that have short roots, not trees or shrubs

- Pump the system to remove sludge every 2 to 5 years depending on the number of people and amount of solids put into the system

- Periodically check the distribution box, if installed, for sludge seepage (seepage requires a check of the tank and filter)

- Periodically check for even flow through all lines (heaving frost and settling can have an effect on the load each line receives)